Adaptive Predictive Load Protection

The Idea

We are building a system that learns how each appliance normally consumes electricity and uses that information to predict abnormal or unsafe operation.

The system continuously monitors power, current, voltage, power factor, and operating time. Instead of applying the same fixed threshold to every appliance, it builds a separate electrical profile for each one.

How It Works

When an appliance is used for the first time, the system learns its electrical fingerprint, including:

- Startup power and peak current

- Startup duration

- Nominal power and current

- Power factor

- Typical operating behavior

This information is stored as an appliance profile.

When the same appliance is used again, its new measurements are compared with its historical profile and the profile is updated.

Since only one appliance operates at a time, the measured electrical behavior can be directly associated with the active appliance.

Mathematical Model

The system uses simple statistical calculations to understand what is normal for each appliance.

It calculates the mean and standard deviation of its learned behavior. A simplified dynamic threshold is:

T = P_normal + kσ + M

Here, P_normal is the learned normal power, σ represents normal variation, k controls sensitivity, and M is a configured safety margin.

The threshold is then constrained by the configured hard safety limit. This makes the threshold data-driven and appliance-specific rather than a single arbitrary value.

Dynamic Threshold

The threshold considers:

Hard Safety Limit + Learned Appliance Behavior + Current Load + Historical Behavior + Degradation

For example, a laptop may normally operate around 90 W while a heater may operate around 2,000 W. Their intervention thresholds therefore should not be identical.

The threshold is continuously refined as the system gains more information about the appliance.

Appliance Degradation

Because the system keeps historical profiles, it can detect persistent changes in an appliance's electrical behavior.

Normal startup: 800 W

Later: 1000 W

Later: 1100 W

Later: 1200 W

A consistent increase can be flagged as abnormal behavior or a possible indication of appliance degradation.

The system looks for persistent trends rather than reacting to a single unusual measurement.

Predictive Protection

The system combines the appliance's current behavior with its learned history to calculate a risk level.

If the load approaches or exceeds the dynamically calculated intervention threshold for a sufficient period, the ESP32 disconnects the relay.

- Learn appliance

- Build electrical profile

- Compare current behavior with history

- Detect abnormal behavior / degradation

- Calculate dynamic threshold

- Predict increasing risk

- Relay OFF when intervention is required